K9HZ ANTENNA TUNER – CONTROL BOARD

BUILD INSTRUCTIONS for PCB V2.0

January 20, 2026

Operating Data:

Power Requirements: 12-15 VDC at 100ma max.

Frequency Range:  1.8MHz – 54Mhz

RF Input:  Up to 150W Maximum.

Control:  I2C for commands to impedance match.

Impedance Matching:  35-1,200 Ohms inclusive.

Background

This antenna tuner was

Inventory and Prework

The full BOM is given in Table 1.  Note that the full BOM parts should not be ordered; it is only presented for reference.  Spend some time with the list of options for this board before you order.  Understand what each option does and select the best set of options for your situation.  Once option selection is made, order and inventory your parts.

Board Option:

There are two options to that are selectable at build time for this tuner.

- Line Section

The first option is the inclusion of a line section for determining forward power, reflected power, and therefore SWR.  You can build and use this option if you are using the tuner with an earlier version of the T41 (V10 or V11), or your radio does not use either the K9HZ LPF or the line section on the K9HZ LPF (If this is the case, its better to use the line section on the ,

1.8Mhz HPF.

1.	Build.

2.	Bypass (for MF reception).  Leave C14, C15, C16, L1, and L2 off the board.  Put 0 Ohm resistors in positions for C14 and C16, and a 0.1uf 50V capacitor in position C15.

FOR and REF Power.

- Detected by diodes:

  - Leave C139, C147, C144, C137, C152, C154, C155, C150 L36, L41, R16, R17, R18, R22, R24, R25, U14, U19 off the board.

- Detected by AD8307’s:

  - Leave C149, C156, D1, D2, R19, R20, R26, R27, R28, R29 off the board.

- FOR and REF Sent via I2C using the A/D:

  - Leave R23 and R30 off the board.

- FOR and REF analog signals sent via BANDS connector:

  - Leave C145, C146, C151, U17, and U20 off the board.

WORK IN PROGRESS>…