K9HZ 100W LPF – FILTER BOARD

BUILD INSTRUCTIONS for PCB V1.10

JUNE 11, 2026

The K9HZ LPF board set is intended to be a very compact yet effective 11 band low-pass filter network to meet FCC regulations.  While this board was intended to be use in a set of three, it can be used stand-alone for just using the filters.

Operating Data

Power Requirements: 	12-15 VDC at 100ma max (provided by the Control Board).

Frequency Range: 		1.8MHz – 54MHz/ 160M – 6M in 11 bands inclusive.

RF Input:  			150W Maximum.  Input and Output with bypass.

Control:  			BCD band data from the Control Board via two connectors.

Performance:			N-th order harmonic reduction more than than 43 db.

Inventory and Prework

Spend time to determine if you want all 11 bands for 1.8MHz-54MHz/ 160M-6M (all amateur bands included in that frequency range).  If you wish to delete a particular band or build that filter later, just remove those parts from the BOM before ordering.  The 100W BOM is:

The filter board can also be built as a 20W maximum board for QRP operations.  The BOM for that combination of parts is listed in the BOM directory.

Keep the capacitors in their envelopes until you need them as they are hard to discern if you mix them up.  A good 45W soldering pencil, solder, flux, and a good set of forceps are all that is needed to complete this board.  Begin by wiping the board, front and back, with Isopropyl alcohol.  This will make soldering parts on the board easier.

Building the LPF Filter board – Pre Assembly Tasks

Spend time winding the torroids before building the board.  If you have an LCR meter or a VNA, wind each inductor… measure it, and label it.  The number of turns given in the BOM is a very good starting point and should work sufficiently as they have been verified in our quality control process.  Spread the windings evenly over the cores.  If you don’t have a way to measure the inductance of the wound inductor, stay with the number of turns suggested in the BOM.  Wind the cores only for the bands you want to use with your finished radio (be warned that it may be difficult to add the inductors, capacitors, and relays, for bands omitted in the initial build).

The following is a table that associates the inductors, capacitors, and relays to each associated band.  You don’t have to install parts in this table for the bands you never intend to use (that is, omit them!).  The bypass position (K32, K33) will be used when no filter is in that band position, programmable in software.

Sequence of Component Placement

The order of component placement on the LPF filter board is critical.

- All capacitors (except those you have determined are associated with bands which you will never transmit.. as from the table above) should be populated FIRST.  Capacitors are in BOTH sides of the board.

- Next, start in the CENTER of the board and alternate placing RELAYS ON THE BOTTOM and INDUCTORS ON THE TOP in a circle expanding out to the edges to allow soldering the relays and inductors.  There is no exact way to do this, but it will become obvious how to proceed as you progress.  The relays are harder to place and solder once the inductors are in place, so place a few relays first.  Leave relays K32 and K33 for the next step.

- Once relays (except for K32 and K33) and inductors are soldered in place, place and solder all connectors in place.

- Finally, solder K32 and K33 in place.

- This concludes building the LPF Filter module.  Clean the board with IPA.

- The Filter Board can be tested using the test algorithms provided on the software GITHUB.